04 USB Camera

## Introduction

The USB camera is a digital camera that is connected to a computer or embedded device by means of the General Line bus (USB). It is capable of transmitting captured video signals or images to the host in real time for video calls, surveillance, image collection or computer visual applications.

## Hardware readiness

Nvidia Jetson Orin Nano Super Kit (jetpack 6.2 system installed)

Helconway, 1080p USB camera.

## Hardware Connection

USB cameras have plug-in features, so they can be detected by connecting them directly to the USB interface at Jetson.

ls /dev/video*

Linux UVC (USB Video Class) drives to create different nodes for the camera's main and auxiliary video stream.

The default main video stream is /dev/video0, which is secondary or compressed.

## Camera Preview

Install ffmpeg

sudo apt update sudo apt install ffmpeg -y

Specify resolution and frame to open the camera

ffplay -f v4l2 -framerate 30 -video_size 640x480 /dev/video0