05 UART Serial Communication

## Introduction

The UART serial communication on Jetson is a standard serial communication method achieved through the development of a 40-Pin extension or panel-mounted special interface, which can be used to connect equipment such as external MMUs (e.g. Arduino), sensors, GPS modules or top machines.

## Hardware Connection

Jetson's serial entry in pin8 and pin10

Pin 8 and pin 10 will be tested back in loop using the Dupont line.

## Serial Test

Use cutecom to validate the GPS serial function. Implements the following code installation and starts cutecom in the jetson terminal.

sudo apt update sudo apt install cutecom -y sudo cutecom

Opens the /dev/ttyTHS1 serial number with cutecom click open: Enter the data to be sent at Input, the window above shows the data sent by the /dev/ttyTHS1 and the window below shows the data received by /dev/ttyTHS1.

If /dev/ttyTHS1 cannot be opened, the terminal runs Sudo Chmod 666 /dev/ttyTHS1 gives permission.