Naming Conventions

For example, the naming of 8GB RAM and 32GB eMMC CM5 module with Hailo-8 26TOPS AI Accelerator function tailored for edge AI computer is reComputer Industrial R2035-12.

Revision History

| Version | Date | Description |
| --- | --- | --- |
| 1.0 | 2025-07-14 |  |

Contents

Naming Conventions	0

Revision History	0

C1. Introduction	1

1.1 Overview	1

1.2 Feature	1

1.3 Specification	2

1.4 Dimension	4

C2. Hardware Overview	5

2.1 System Overview	5

2.1.1 Interface Overview	5

2.1.2 Mainboard Overview 	6

2.1.3 Power Diagram	7

2.1.4 Block Diagram	8

2.1.5 IIC Diagram	9

2.2 Interface Description	10

2.2.1 LED Indicator Status	10

2.2.2 Buzzer	12

2.2.3 RS485	12

2.2.4 RS232	13

2.2.5 DI	14

2.2.6 DO	15

2.2.7 Boot Switch	16

2.2.8 USB	16

2.2.9 SIM Slot(Internal)	16

2.2.10 M.2 M-KEY Slot	17

2.2.11 Mini-PCIe  Slot	18

2.2.12 M.2 B-KEY  Slot	18

2.2.13 Reset Hole	19

2.2.14 Ethernet RJ45	19

2.2.15 HDMI	19

2.2.16 RTC	19

2.2.17 Watchdog	20

2.3 Optional Interfaces and Module	20

2.3.1 Wi-Fi/BLE	21

2.3.2 4G Module	21

2.3.3 5G Module	21

2.3.4 LoRa® Module	21

2.3.5 Zigbee Module	22

2.3.6 SSD	22

2.3.7 Encryption Chip TPM 2.0	22

2.3.8 UPS	23

C3. Configuring System	24

3.1 Flashing Image	24

3.2 Query GPIO Mappings	27

3.3 USER LED Testing	27

3.4 SPI Communication Testing	27

3.5 Wi-Fi Scanning	28

3.6 Bluetooth Scanning	28

3.7 LoRa® over Mini-PCIe	28

3.7.1 LoRa® SPI	28

3.7.2 LoRa® USB	32

3.8 5G Cellular over M.2 B-KEY	33

3.9 4G Cellular over M.2 B-KEY	35

3.10 Zigbee over Mini-PCIe	36

3.11 RS485 Testing	36

3.12 RS232 Testing	38

3.13 DI Testing	39

3.14 DO Testing	40

3.15 USB Hub Testing	41

3.16 RTC 	41

3.17 Watchdog	41

3.18 Buzzer	43

3.19 TPM 2.0	43

3.20 ATECC608A	43

3.21 EEPROM	44

3.22 SSD	44

3.23 UPS for Safe Shut Down	44

3.24 AI  Accelerator	48

3.25 Installing Ubuntu on reComputer Industrial R2000	49

3.26 Customized Linux: Yocto and Mender	49

3.27 Costomized Linux: Buildroot	50

C4. Assembly Guide	51

4.1 Disassembly Guide	51

4.2 Assemble Nano SIM Card	51

4.3 Assemble SSD	52

4.4 Assemble M.2 AI  Accelerator	52

4.5 Assemble 5G/4G/LoRa®/Zigbee Module and Antenna	53

4.6 Assemble TPM 2.0 Module	54

4.7 Assemble UPS module	54

4.8 Mounting Guide	55

C5. Accessories List	56

C6. Warranty & Support	57

6.1 Warranty	57

6.2 Support	57

C1. Introduction

  - 1.1 Overview

The reComputer Industrial R2000, powered by Raspberry Pi CM5, is an adaptable edge AI/IOT conrollert  with AI capabilities. It features comprehensive industrial interfaces (2x Ethernet, 4xUSB, 3x RS485, 1x RS232, 8x DI and 8x DO) and flexible wireless connectivity options (4G/5G, LoRa®, Wi-Fi/BLE), making it ideal for diverse industrial applications.

Application

The reComputer Industrial  R2000 series is ideal for use in industrial AI applications. It can be used in aspects such as data acquisition and process monitoring, automation and robot control, intelligent manufacturing, and industrial communication and networking. With its small size, flexibility, low cost, and programmability, it provides strong support for automation & IoT system and more.

  - 1.2 Feature

Industrial-Grade Reliability

- Fanless compact PC with wide temperature support -20°C to 65°C

- Hardware watchdog

- UPS Supercapacitor(optional)

- High-quality metal case, compatible with DIN-rail and Wall installation

- Production Lifetime: reComputer Industrial R2000 will remain in production until at least December 2036

High-Efficiency AI Computing

- Powered by Raspberry Pi CM5

- Broadcom BCM2711 quad-core Cortex-A72 (ARM v8) 64-bit SoC @ 1.5GHz

- Up to 16GB RAM and 64GB eMMC

- Powered by Hailo-8 AI accelerator up to 26 TOPS(optional)

Rich Wireless Capabilities

- On-chip Wi-Fi

- On-chip BLE

- Mini-PCIe: LTE, USB LoRa®, USB Zigbee

- M.2 B-KEY: 4G/5G

Rich Interfaces

- 3x RS485 (isolated), 1x RS232 (isolated), 8x isolated DI ports, 8x isolated DO ports

- 1x 10M/100M/1000M Ethernet (Support PoE PD)

- 1x 10M/100M Ethernet

- 2x HDMI 2.0

- 3x Type-A USB3.0

- 1x Type-A USB2.0

- 1x Type-C USB2.0 (For flashing OS & Debug)

- 1x Nano SIM card slot

  - 1.3 Specification

| Parameter | Description |  |
| --- | --- | --- |
| Hardware Specification |  |  |
| Product Series | R20xx-12 | R20xx-10 |
| CPU | Raspberry Pi Compute Module 5, 2.4GHz quad-core 64-bit Arm Cortex-A76 |  |
| AI Processor | Pre-installed 1x Hailo-8 M.2 AI Acceleration 26TOPS | * |
| Operating System | Raspbian, Debian |  |
| RAM | 2GB/4GB/8GB/16GB |  |
| eMMC | 16GB/32GB/64GB |  |
| System Specification |  |  |
| Power Input | DC 9V~36V, 2-pin Terminal Block |  |
| PoE(as powered device) | IEEE 802.3at Standard 25.5W PoE |  |
| Power Switch | No |  |
| Reboot Switch | Yes |  |
| Interface |  |  |
| Ethernet | 1 x 10/100/1000 Mbps(supports PoE PD) |  |
|  | 1 x 10/100 Mbps IEEE 802.3/802.3u |  |
| USB | 3 x USB-A 3.0 Host; 1 x USB-A 2.0 Host |  |
|  | 1 x USB-C 2.0 (For flashing OS & Debug) |  |
| RS485 | 3x RS485(Isolated) |  |
| RS232 | 1x RS232(Isolated) |  |
| DI | 8 x Isolated DI Ports |  |
|  | Input Voltage: 5~24V DC |  |
| DO | 8 x Isolated DO Ports |  |
|  | Output Voltage: <60V DC |  |
| SIM Card | 1x Nano SIM Card Slot |  |
| M.2 Slot | 1x M.2 M-KEY 2280 Slot for ;NVMe SSD； 1x M.2 M-KEY 2280 Slot for AI Accelerator 1x M.2 NVMe Slot, t, M.2 M-KEY 2280 (Pre-installed 1x Hailo-8 M.2 AI Acceleration 26TOPS); 1x M.2 B-KEY 3042/3052 Slot for 5G/4G LTE; | 1x M.2 M-KEY 2280 Slot for NVMe SSD； 1x M.2 M-KEY 2280 Slot for AI Accelerator; 1x M.2 B-KEY 3042/3052 Slot for 5G/4G LTE 2x M.2 NVMe Slot, t, M.2 M-KEY 2280; 1x M.2 NVMe Slot, t, M.2 B-KEY |
| Mini-PCIe | 1x Mini-PCIe for LoRa module |  |
| LED | 4 x LED indicators |  |
| Buzzer | 1 |  |
| Reset Button | 1 |  |
| HDMI | 2 x HDMI 2.0 |  |
| Wireless Communication |  |  |
| Wi-Fi 2.4/5.0 GHz | On-chip Wi-Fi |  |
| BLE 5.0 | On-chip BLE |  |
| LoRa® | USB LoRa®*/SPI LoRa®* |  |
| 4G/5G Cellular | 4G LTE/5G* |  |
| Zigbee | USB Zigbee* |  |
| Standards |  |  |
| EMC | ESD: EN61000-4-2, Level 3 |  |
|  | EFT: EN61000-4-4, Level 2 |  |
|  | Surge: EN61000-4-5, Level 2 |  |
| Certification | CE, FCC |  |
|  | TELEC |  |
|  | RoHS |  |
|  | REACH |  |
| Ambient Conditions |  |  |
| Ingress Protection | IP40 |  |
| Operating Temperature | -20~65 °C |  |
| Operating Humidity | 10~95% RH |  |
| Storage Temperature | -40~85 °C |  |
| Others |  |  |
| Supercapacitor UPS | SuperCAP UPS LTC3350 Module* |  |
| Hardware Watchdog | 1~255s |  |
| RTC | High Accuracy RTC |  |
| Security | Encryption Chip TPM 2.0* |  |
|  | ATECC608A |  |
| Heat Dissipation | Fanless |  |
| Warranty | 2 years |  |
| Production Lifetime | Until December 2036 |  |
| Statement | Options marked with * require additional purchase according to the accessories list. |  |

| Component and Interface Status Statement |  |
| --- | --- |
| Reserved | Designated for future use or expansion. |
| Optional | Non-essential components, users can choose to include or exclude. |
| Occupied | Currently in use and integral to product functionality. |
| Included | Essential components provided with standard package. |

  - 1.4 Dimension

| Mechanical |  |
| --- | --- |
| Dimension(W x H x D) | 197.5 mm x 155 mm x 67.5 mm |
| Enclosure | 6063 aluminum alloy heat sink, 5052 aluminum alloy enclosure |
| Mounting | DIN-rail/Wall |
| Weight(Net) | 1280g |

C2. Hardware Overview

  - 2.1 System Overview

## 2.1.1 Interface Overview

## 2.1.2 Mainboard Overview

## 2.1.3 Power Diagram

The reComputer Industrial R2000 supports two power supply options: DC terminal and PoE port. This provides flexibility in power supply selection and allows for easy integration with various power sources.

2-Pin Power Terminal

The reComputer Industrial R2000 is supplied with a terminal DC voltage of 9~36V. The power supply is connected via the 2-pin power terminal block connector.

To ground the reComputer Industrial R2000, the ground wire can be secured to the screw located at the right of the power terminal.

PoE

The ETH0(PoE PD) port of reComputer Industrial R2000 can support PoE power supply, providing a convenient and efficient way to power the device over Ethernet. This option simplifies the installation process and reduces the amount of cabling required, making it an ideal solution for applications with limited power sources or where power outlets are not readily available.

- PoE PD: IEEE 802.3at, 25.5W Max

Note

It's worth noting that the PoE module provided with the reComputer Industrial R2000 is compliant with the IEEE 802.3at standard and can provide a maximum power supply of 25W. Therefore, if there is a need to connect high-power peripherals such as 5G or 4G modules, the PoE power supply may not be sufficient. In this case, it's recommended to use the DC terminal for power supply instead to ensure stable and reliable operation of the device.

Power On and Power Off

The reComputer Industrial R2000 does not come with a power button by default, and the system will automatically start up once power is connected. When shutting down, please select the shutdown option in the operating system and wait for the system to fully shut down before cutting off power. To restart the system, simply reconnect to the power.

## 2.1.4 Block Diagram

## 2.1.5 IIC Diagram

  - 2.2 Interface Description

| Interface |  |
| --- | --- |
| Ethernet | 1 x 10/100/1000 Mbps (supports POE PD) |
|  | 1 x 10/100 Mbps IEEE802.3/802.3u |
| USB | 3 x USB-A 3.0 Host; 1 x USB-A 2.0 Host |
|  | 1 x USB-C 2.0 (for flashing OS and Debug) |
| RS485 | 3x RS485(Isolated) |
| RS232 | 1x RS232(Isolated) |
| DI | 8 x Isolated DI Ports |
|  | Input Voltage: 5~24V DC |
| DO | 8 x Isolated DO Ports |
|  | Output Voltage: <60V DC |
| HDMI | 2 x HDMI 2.0 |
| SIM Card Slot | supports Nano SlM Card |
| M.2 M-KEY Slot | 1x 2280 for M.2 NVMe SSD |
|  | 1x 2280 for AI Accelerator |
| M.2 B-KEY Slot | 1x for 5G/4G LTE |
| mini PCIe Slot | 1x for 4G LTE/LoRa/Zigbee |
| LED | 4 x LED indicators |
| Buzzer | 1 |
| Reset Button | 1 |

## 2.2.1 LED Indicator Status

The reComputer Industrial R2000 features 4 LED indicators that serve to signal the machine's operational status. Please refer to the table below for the specific functions and status of each LED:

| LED Indicator | Color | Status | Description |
| --- | --- | --- | --- |
| PWR | Green | On | The device has been connected to power. |
|  |  | Off | The device is not connnected to power. |
| ACT | Green |  | Under Linux this pin will flash to signify eMMC access. If any error occurs during booting, then this LED will flash an error pattern which can be decoded using the look up table(Raspberry Pi Documentation - Configuration) on the Raspberry Pi website. |
| USER | Green/Red/Blue |  | Need to be defined by user. |
| 4G/5G | Green | On | The dial-up is successful and the connection is normal. |
|  |  | Off | 4G/5G signal is not connected or the device is not powered on. |

ACT Status table

| Long flashes | Short flashes | Status |
| --- | --- | --- |
| 0 | 3 | Generic failure to boot |
| 0 | 4 | start*.elf not found |
| 0 | 7 | Kernel image not found |
| 0 | 8 | SDRAM failure |
| 0 | 9 | Insufficient SDRAM |
| 0 | 10 | In HALT state |
| 2 | 1 | Partition not FAT |
| 2 | 2 | Failed to read from partition |
| 2 | 3 | Extended partition not FAT |
| 2 | 4 | File signature/hash mismatch - Pi 4 |
| 4 | 4 | Unsupported board type |
| 4 | 5 | Fatal firmware error |
| 4 | 6 | Power failure type A |
| 4 | 7 | Power failure type B |

If the ACT LED blinks in a regular four blink pattern, it cannot find bootcode(start.elf).

If the ACT LED blinks in an irregular pattern then booting has started.

If the ACT LED doesn't blink, then the EEPROM code might be corrupted, try again without anything connected to make sure. For more detail please check the Raspberry Pi forum:

STICKY: Is your Pi not booting? (The Boot Problems Sticky) - Raspberry Pi Forums.

For more detail please check the Raspberry Pi forum: https://forums.raspberrypi.com//viewtopic.php?f=28&t=58151

USER Indicator

The reComputer Industrial R2000 contains a USER indicator, and users can customize the status according to actual needs.

Note

For details of USER LED testing, please refer to section 3.3.

## 2.2.2 Buzzer

The reComputer Industrial R2000 contains an active buzzer, which can be used for various purposes such as alarm and event notifications. Enter in the terminal of reComputer Industrial R2000:

cat /sys/kernel/debug/gpio

This command will output the GPIO corresponding to the Buzzer_EN is gpio627.

Note

For details of buzzer testing, please refer to section 3.18.

## 2.2.3 RS485

The reComputer Industrial R2000 series equipment includes 3x RS485 ports, 6-Pin 3.5mm spacing phoenix terminals.

The silkscreen of single RS485 is "A/B/GND".

Pin Definition

Terminal pins are defined as follows:

|  | Pin ID | Pin Name |
| --- | --- | --- |
|  | 1 | R485-2_A |
|  | 3 | R485-2_B |
|  | 5 | R485-2_GND |
|  | 7 | R485-3_A |
|  | 9 | R485-3_B |
|  | 11 | R485-3_GND |
|  | 8 | R485-4_A |
|  | 10 | R485-4_B |
|  | 12 | R485-4_GND |

Connecting Cables

Schematic diagram of RS485 wires is as follws:

Note

The product packaging contains some 120 Ohm terminal resistors. You can use them as needed when communicating via RS485. For details of RS485 testing, please refer to section 3.11.

## 2.2.4 RS232

The reComputer Industrial R2000 series equipment includes 1x RS232 ports, 6-Pin 3.5mm spacing phoenix terminals.

The silkscreen of single RS232 is "TX/RX/GND".

Pin Definition

Terminal pins are defined as follows:

|  | Pin ID | Pin Name |
| --- | --- | --- |
|  | 2 | R232-1_TX |
|  | 4 | R232-1_RX |
|  | 6 | R232-1_GND |

Connecting Cables

Schematic diagram of RS232 wires is as follws:

Note

For details of RS232 testing, please refer to section 3.12.

## 2.2.5 DI

The reComputer Industrial R2000 series equipment includes 8x DI ports, 3-Pin 3.5mm spacing phoenix terminals.

The silkscreen of single DI is "DI/G_DI".

Pin Definition

Terminal pins are defined as follows:

|  | Pin ID | Pin Name |
| --- | --- | --- |
|  | 13 | DI1 |
|  | 15 | DI2 |
|  | 17 | DI3 |
|  | 19 | DI4 |
|  | 21 | DI5 |
|  | 23 | DI6 |
|  | 25 | DI7 |
|  | 27 | DI9 |
|  | 29 | G_DI |

Connecting Cables

Schematic diagram of a single DI wires is as follws:

| Parameter | Description |
| --- | --- |
| Input Type | PNP |
| Isolation Protection | 5 kV |
| DI to G_DI | ON state: 5~30 VDC |

Note

For details of DI testing, please refer to section 3.13.

## 2.2.6 DO

The reComputer Industrial R2000 series equipment includes 8x DO ports, 3-Pin 3.5mm spacing phoenix terminals.

The silkscreen of single DO is "DO/G_DO".

Pin Definition

Terminal pins are defined as follows:

|  | Pin ID | Pin Name |
| --- | --- | --- |
|  | 14 | DO1 |
|  | 16 | DO2 |
|  | 18 | DO3 |
|  | 20 | DO4 |
|  | 22 | DO5 |
|  | 24 | DO6 |
|  | 26 | DO7 |
|  | 28 | DO8 |
|  | 30 | G_DO |

Connecting Cables

Schematic diagram of a single DO wires is as follws:

| Parameter | Description |
| --- | --- |
| Onput Type | Transistor |
| Isolation Protection | 5 kV |
| Output | < 60V DC |

Note

For details of DO testing, please refer to section 3.14.

## 2.2.7 Boot Switch

The Boot Switch of the reComputer Industrial R2000 is connected to the nRPI_BOOT pin of CM5. This switch provides users with the option to select the boot source between eMMC and USB. In normal mode, the switch should be set away from the side with the "BOOT" label, enabling the system to boot from eMMC. Conversely, when users need to flash the system image, they should set the switch towards the "BOOT" label, allowing the system to boot from the Type-C USB interface.

| Switch Position | Mode | Description | nRPI-BOOT |
| --- | --- | --- | --- |
|  | Normal mode | Boot from eMMC | Low |
|  | Flash mode | Boot from USB | High |

## 2.2.8 USB

The reComputer Industrial R2000 is equipped with 1x USB Type-C port and 4x USB Type-A ports. Please refer to the table below for their functions and descriptions.

| Type | Quantity | Protocol | Function | Description |
| --- | --- | --- | --- | --- |
| Type-C | *1 | USB2.0 | USB-Device | Used for serial port debugging, burning image, etc. |
| Type-A | *1 | USB2.0 | USB-Host | Connect different USB devices such as flash drives, USB keyboards or mouses. |
| Type-A | *3 | USB3.0 | USB-Host | Connect different USB devices such as flash drives, USB keyboards or mouses. |

## 2.2.9 SIM Slot(Internal)

The reComputer Industrial R2000 series equipment includes an internal Nano SIM card slot, which is used to install Nano SIM card for obtaining 5G/4G signals.

The size differences between standard SIM, Micro SIM and Nano SIM cards are as follows:

Note

Please note that the standard version of reComputer Industrial R2000 does not come with a 5G/4G module. If you require 5G/4G functionality, an additional 5G/4G module must be purchased separately. For more information, please refer to section "2.3.2 4G Module".

## 2.2.10 M.2 M-KEY Slot

| Slot | Supported Protocol |
| --- | --- |
| M.2 M-KEY 2280 | M.2 NVMe SSD |
| M.2 M-KEY 2280 | M.2 AI Accelerator |

The M.2 M-KEY 2280 slot on the reComputer Industrial R2000 is designed to accommodate NVMe M.2 2280 SSDs for 128GB, 256GB, 512GB, 1TB and 2TB in capacity. This slot allows for high-speed storage expansion, enabling users to enhance the performance and capacity of their system.

Note

There are two main uses for SSD cards: 1. High Capacity Storage: SSD cards can be utilized for high-capacity storage needs. 2.Boot Drive with Image: Another usage involves using the SSD both as a high-capacity storage and for storing system images, allowing booting directly from the SSD card. It's important to note that not all SSD cards available in the market support the second usage. Therefore, if you intend to use it as a boot drive and are unsure about which model to purchase, we recommend opting for our recommended 2TB SSD(SKU 114993467). This model has been tested and verified for boot functionality, reducing the risk of compatibility issues and minimizing trial and error costs.

The M.2 M-KEY 2280 slot on the reComputer Industrial R2000 is designed to accommodate PCIE M.2 AI  Accelerator. And the R20xx-12 series has been pre-installed with a Hailo-8 M.2 AI Acceleration up to 26TOPS.

## 2.2.11 Mini-PCIe  Slot

The MiniPCIe slot on the reComputer Industrial R2000 is designed to accommodate devices such as 4G LTE, USB LoRaWAN®, and USB Zigbee.

## 2.2.12 M.2 B-KEY  Slot

The M.2 B-KEY slot on the reComputer Industrial R2000 is designed to accommodate 5G/4G LTE.

## 2.2.13 Reset Hole

There is a Mini Push Button Switch located in the reset hole of reComputer Industrial R2000. By pressing this button with a thin object, the CM5 can be reset. This pin when high signals that the CM5 has started. Driving this pin low resets the module

## 2.2.14 Ethernet RJ45

| Name | Type | Speeds | PoE PD |
| --- | --- | --- | --- |
| ETH0 | CM5 native Gigabit Ethernet | 10/100/1000 Mbit/s | Supported |
| ETH1 | Converted from USB | 10/100 Mbit/s | Not Supported |

The reComputer Industrial R2000 comes with two Ethernet RJ45 ports. ETH0 is a CM5 native Gigabit Ethernet interface that supports three different speeds: 10/100/1000 Mbit/s. It can enable power-over-Ethernet (PoE) delivery through this interface, providing power to the reComputer Industrial R2000. Another one ETH1 supports 10/100 Mbit/s which is converted from USB.

Note

For more detail about PoE, please check section "2.3.5 PoE".

## 2.2.15 HDMI

The reComputer Industrial R2000 features 2x native HDMI interface from CM5, supporting up to 4K @ 60 fps video output. It is ideal for applications that require multiple displays, allowing users to output their content to external large screens.

## 2.2.16 RTC

The reComputer Industrial R2000 features an RTC circuit that powered by the capacitor, enabling it to maintain timekeeping functionality even in the event of power loss.

Note

For details of RTC testing, please refer to section 3.16.

## 2.2.17 Watchdog

The reComputer Industrial R2000 comes equipped with an independent hardware watchdog circuit that ensures automatic system reboot in case of abnormal system crashes. The watchdog circuit is implemented through RTC and allows for flexible feeding times from 1 to 255 seconds.

Note

For details of watchdog testing, please refer to section 3.17.

  - 2.3 Optional Interfaces and Module

The reComputer Industrial R2000 supports a rich selection of expansion modules and accessories, making it suitable for a wide range of scenarios and requirements. If you are interested in customizing the reComputer Industrial R2000, please contactodm@seeed.cc for more information.

Here is the accessories and optional modules list:

| Remark | Item | Product Name | SKU |
| --- | --- | --- | --- |
| Must be used together for LoRaWAN® Function | LoRa® Module | Region optional LoRaWAN® Gateway Module(SPI)-US915 | 114992969 |
|  |  | Region optional LoRaWAN® Gateway Module(USB)-US915 | 114992629 |
|  |  | Region optional LoRaWAN® Gateway Module(USB)-US915 | 114992991 |
|  |  | Region optional LoRaWAN® Gateway Module(SPI)-EU868 | 114993268 |
|  |  | Region optional LoRaWAN® Gateway Module(SPI)-EU868 | 114992549 |
|  |  | Region optional LoRaWAN® Gateway Module(USB)-EU868 | 114992628 |
|  | LoRa® Antenna | LoRa® Antenna Kit - 868-915 MHz | 110061501 |
|  | Zigbee Module | Mini-PCIe USB Zigbee Module | 110992005 |
|  | Zigbee Antenna | Zigbee Antenna Kit for reComputer R | 110061641 |
| 4G antenna with 4G module for 4G function, GPS antenna with 4G module for GPS function | 4G module | LTE Cat 4 EC25-AFXGA-Mini-PCIe Module - for North American | 113991134 |
|  |  | LTE Cat 4 EC25-EUXGR-Mini-PCIe Module - for EMEA and Thai | 113991135 |
|  |  | LTE Cat 4 EC25-AUXGR-Mini-PCIe Module - for Australia | 113991174 |
|  |  | LTE Cat 4 EC25-EFA-Mini-PCIe Module - for Thai | 113991214 |
|  |  | LTE Cat 4 EC25-EMGA-Mini-PCIe Module - for Malaysia | 113991234 |
|  |  | LTE Cat 4 EC25-JFA-mini-PCIe | 113991296 |
|  | 4G Antenna | 4G Antenna Kit for 4G module | 110061502 |
|  | GPS Antenna | GPS Antenna Kit for EC25 4G Module | 110061521 |
|  | Encryption Chip TPM 2.0 | TPM 2.0 Module with infineon SLB9670 | 114993114 |
|  | SSD card | NVMe M.2 2280 SSD 2TB | 114993467 |
|  |  | NVMe M.2 2280 SSD 1TB | 112990267 |
|  |  | 512GB NVMe M.2 PCle Gen3x4 2280 Internal SSD | 112990247 |
|  |  | 256GB NVMe M.2 PCle Gen3x4 2280 Internal SSD | 112990246 |
|  |  | 128GB NVMe M.2 PCle Gen3x4 2280 Internal SSD | 112990226 |
|  | UPS | SuperCAP UPS LTC3350 Module | 110992004 |

The reComputer Industrial R2000 mainboard features two Mini-PCIe slots. Mini-PCIe slot 1 supports 4G module, LoRa® module using the USB protocol and Zigbee module using USB protocol; while Mini-PCIe slot 2 supports LoRa® module using the USB and SPI protocol and Zigbee module using USB protocol. Additionally, 4G module and LoRa® module shouldn't be used at the same time, can not plug in two LoRa® modules on board.

Note

Can not plug in 2 LoRa® modules on board.

## 2.3.1 Wi-Fi/BLE

The  reComputer Industrial R2000 is powered by the CM5 with an onboard Wi-Fi/BLE version, providing the same Wi-Fi/BLE parameters as the CM5. For detailed parameter information, please refer to the Raspberry Pi official website.

## 2.3.2 4G Module

The reComputer Industrial R2000 mainboard features one M.2 B-KEY Slot  and one Mini-PCIe Slot, Both M.2 B-KEY Slot and  Mini-PCIe slot  supporting a 4G module. The EC25 4G module from Quectel has been fully tested to be compatible with the reComputer Industrial R2000.

Note

Please note that if you require 4G functionality, it is necessary to purchase the corresponding 4G module and external antenna, and follow the instructions in section 4.5 "Assemble 4G/LoRa®/Zigbee Module and Antenna".

## 2.3.3 5G Module

The reComputer Industrial R2000 mainboard features one M.2 B-KEY Slot, supporting a 4G/5G module using the USB protocol. The EC25 4G module from Quectel has been fully tested to be compatible with the reComputer Industrial R2000.

Note

Please note that if you require 4G/5G functionality, it is necessary to purchase the corresponding 4G module and external antenna, and follow the instructions in section 4.5 "Assemble 4G/LoRa®/Zigbee Module and Antenna".

## 2.3.4 LoRa® Module

The Mini-PCIe slot supports LoRa® module using the USB and SPI protocol.  The WM1302 module from Seeed Studio has been fully tested to be compatible with the reComputer Industrial R2000.

Note

Please note that if you require LoRa® functionality, it is necessary to purchase the corresponding LoRa® module and external antenna, and follow the instructions in section 4.5 "Assemble 4G/LoRa®/Zigbee Module and Antenna".

## 2.3.5 Zigbee Module

The Mini-PCIe slot offer support for Zigbee modules utilizing the USB protocol, allowing for seamless integration of Zigbee functionality into compatible devices. This feature enables efficient communication and control within Zigbee networks, enhancing the versatility and connectivity of the system. With the Mini-PCIe slot available for Zigbee modules, users have the flexibility to implement diverse applications for enhanced reliability.

Note

Please note that if you require Zigbee functionality, it is necessary to purchase the corresponding Zigbee module and external antenna, and follow the instructions in section 4.5 "Assemble 4G/LoRa®/Zigbee Module and Antenna".

## 2.3.6 SSD

The reComputer Industrial R2000 supports 2280 NVMe SSD through the use of a PCIe slot(J7). It is important to note that the CM5's PCIe is gen2.0 with a maximum theoretical speed of 5Gbps. If you are using a Gen3.0 or higher SSD, it may not be able to achieve the SSD's maximum speed. After testing, the reTerminal DM with installed SSD can achieve a maximum write speed of 230MB/s and a maximum read speed of 370MB/s. If you are unsure which SSDs are compatible, you can purchase following the accessories list below.

Note

Please note that: 1- The speed test results may vary depending on the SSD model, testing method, and testing environment. The values provided here are for reference purposes only and were obtained in Seeed's laboratory.

Note

There are two main uses for SSD cards: 1.High Capacity Storage: SSD cards can be utilized for high-capacity storage needs. 2.Boot Drive with Image: Another usage involves using the SSD both as a highcapacity storage and for storing system images, allowing booting directly from the SSD card. It's important to note that not all SSD cards available in the market support the second usage. Therefore, if you intend to use it as a boot drive and are unsure about which model to purchase, we recommend opting for our recommended 1TB SSD(SKU 112990267). This model has been tested and verified for boot functionality, reducing the risk of compatibility issues and minimizing trial and error costs.

## 2.3.7 Encryption Chip TPM 2.0

The TPM features Infineon’s OPTIGA™ TPM SLB9670 which is compliant to the Trusted Computing Group (TCG) TPM 2.0 specification is recommended as encryption chip to the reComputer Industrial R2000. The chip features an SPI interface applied for port J26 on board, to enable a root of trust for platform integrity, remote attestation, and cryptographic services.

Note

Please refer to section 4.6 "Assemble TPM 2.0 Module" for instruction.

## 2.3.8 UPS

The UPS is 7F, which operates in series. The UPS module is positioned between the DC5V and CM5 components, with a GPIO signal utilized to alert the CPU in the event of a power loss from the 5V supply. Upon receiving this signal, the CPU executes an urgent script before the super capacitor's energy is depleted, initiating a "$ shutdown" command.
The backup duration provided by the UPS heavily relies on the system load. Below are some typical scenarios tested with a CM5 module featuring 4GB RAM, 32GB eMMC storage, and a Wi-Fi module.

| Mode of Operation | Time(s) | Remark |
| --- | --- | --- |
| Idle | 15 | Testing under idle conditions with official driver program loaded |
| Full load of CPU | 6 | stress -c 4 -t 10m -v & |

Note

For UPS function please contact us for more information, and the alarm signal is active LOW.

C3. Configuring System

  - 3.1 Flashing Image

Note

The device comes pre-installed and pre-burned with the system image. After powering on for the first time, you can log in using the following default account: - Username: recomputer - Password: 12345678 It is recommended that you change your password immediately after logging in for security reasons.

Here are the steps to re-burn the image:

- Image Download

Download address: GitHub - Seeed-Studio/pi-gen-expand: This project maintains Raspberry OS Image for Seeed's Raspberry-related products

Select the last one and click the date link to download:

If you encounter network issues, you can use the wget command to download.

- Image Burn

  - Download the rpiboot setup installer by click here to install the necessary drivers and the boot tool.

  - Connect reComputer R2000 the PC via USB Type-C cable.

  - Windows will now find the hardware and install the necessary drivers.

  - Search for rpiboot tool that we installed before and open it.

  - Open file explorer and you will see the eMMC of the Computer Module 5 shown as a USB mass storage device.

  - Download Raspberry Pi Imager software from Raspberry Pi software – Raspberry Pi.

  - Open Raspberry Pi Imager software.

  - Press CTRL + SHIFT + X on the keyboard to open Advanced options window.

  - Select the downloaded image file.

Continue burning the image until it succeeds.

Unplug the USB-C port, press the boot button again, and power on/off. If you can log in to the system normally, the image has been burned successfully and can be used normally.

  - 3.2 Query GPIO Mappings

To query GPIO mappings and offsets, follow these steps:

- Copy and paste the following command to query GPIO mappings:

cat /sys/kernel/debug/gpio

This command will provide you with the necessary information regarding GPIO mappings and offsets whenever needed throughout the process.

  - 3.3 USER LED Testing

We provide LEDs in three colors of red, blue and green for users to use. You can enter the /sys/class/leds/ directory to view：

cd /sys/class/leds/ ls

Use the following command to light up the LED of the corresponding color.

sudo su echo 1 > /sys/class/leds/led-red//brightness echo 1 > /sys/class/leds/led-blue/brightness echo 1 > /sys/class/leds/led-green/brightness

The third led-user LED will light up.

  - 3.4 SPI Communication Testing

To test SPI communication by shorting the TPM module's MISO and MOSI pins, follow these steps:

- Clone the spidev-test repository:

# Don't forget to connect to network before running command git clone https://github.com/rm-hull/spidev-test.git

- Navigate into the spidev-test directory:

cd spidev-test

- Compile the spidev_test.c file:

gcc spidev_test.c -o spidev_test

- Run the spidev_test program with the following command:

./spidev_test -D /dev/spidev10.0 -v -p hello

This command tests SPI communication on the specified SPI device ( /dev/spidev10.0 ) with verbose output ( -v ) and sends the message "hello" ( -p hello ).

By shorting the TPM module's MISO and MOSI pins, you're effectively creating a loopback scenario, where data sent on MOSI is received on MISO. This setup allows you to test SPI communication without an actual device connected.

  - 3.5 Wi-Fi Scanning

To scan for Wi-Fi networks:

sudo iwlist wlan0 scan

This command will list available Wi-Fi networks along with their details.

  - 3.6 Bluetooth Scanning

To scan for Bluetooth devices:

sudo bluetoothctl

This command will open the Bluetooth control interface. From there, you can run additional commands to scan for nearby Bluetooth devices:

scan on

This command will start scanning for nearby Bluetooth devices. You can then use other commands within the bluetoothctl interface to interact with Bluetooth devices, such as pairing or connecting to them.

  - 3.7 LoRa® over Mini-PCIe

## 3.7.1 LoRa® SPI

After install the LoRa® SPI to Mini-PCIe slot 2, can configure LoRa® SPI, follow these steps:

- Clone the SX1302_HAL repository:

cd ~/ git clone https://github.com/Lora-net/sx1302_hal

- Navigate into the cloned directory:

cd sx1302_hal

- Modify the configuration file:

sudo nano ./libloragw/inc/loragw_i2c.h

Change #define I2C_DEVICE "/dev/i2c-1" to #define I2C_DEVICE "/dev/i2c-2".

Press  ctrl+x  to exit, press  y  to save changes, and then press  Enter  to return to the command line page.

- Add the packet_forwarder/reset_lgw.sh file:

sudo nano packet_forwarder/reset_lgw.sh

Add the execution code:

#!/bin/sh # This script is intended to be used on SX1302 CoreCell platform, it performs # the following actions: # - export/unpexort GPIO23 and GPIO18 used to reset the SX1302 chip and to enable the LDOs # - export/unexport GPIO22 used to reset the optional SX1261 radio used for LBT/Spectral Scan # # Usage examples: # ./reset_lgw.sh stop # ./reset_lgw.sh start # GPIO mapping has to be adapted with HW # SX1302_RESET_PIN=632 # SX1302 reset SX1302_POWER_EN_PIN=633 # SX1302 power enable SX1261_RESET_PIN=634 # SX1261 reset (LBT / Spectral Scan) AD5338R_RESET_PIN=623 # AD5338R reset (full-duplex CN490 reference design) WAIT_GPIO() { sleep 0.1 } init() { # setup GPIOs echo "$SX1302_RESET_PIN" > /sys/class/gpio/export; WAIT_GPIO echo "$SX1261_RESET_PIN" > /sys/class/gpio/export; WAIT_GPIO echo "$SX1302_POWER_EN_PIN" > /sys/class/gpio/export; WAIT_GPIO echo "$AD5338R_RESET_PIN" > /sys/class/gpio/export; WAIT_GPIO # set GPIOs as output echo "out" > /sys/class/gpio/gpio$SX1302_RESET_PIN/direction; WAIT_GPIO echo "out" > /sys/class/gpio/gpio$SX1261_RESET_PIN/direction; WAIT_GPIO echo "out" > /sys/class/gpio/gpio$SX1302_POWER_EN_PIN/direction; WAIT_GPIO echo "out" > /sys/class/gpio/gpio$AD5338R_RESET_PIN/direction; WAIT_GPIO } reset() { echo "CoreCell reset through GPIO$SX1302_RESET_PIN..." echo "SX1261 reset through GPIO$SX1302_RESET_PIN..." echo "CoreCell power enable through GPIO$SX1302_POWER_EN_PIN..." echo "CoreCell ADC reset through GPIO$AD5338R_RESET_PIN..." # write output for SX1302 CoreCell power_enable and reset echo "1" > /sys/class/gpio/gpio$SX1302_POWER_EN_PIN/value; WAIT_GPIO echo "1" > /sys/class/gpio/gpio$SX1302_RESET_PIN/value; WAIT_GPIO echo "0" > /sys/class/gpio/gpio$SX1302_RESET_PIN/value; WAIT_GPIO echo "0" > /sys/class/gpio/gpio$SX1261_RESET_PIN/value; WAIT_GPIO echo "1" > /sys/class/gpio/gpio$SX1261_RESET_PIN/value; WAIT_GPIO echo "0" > /sys/class/gpio/gpio$AD5338R_RESET_PIN/value; WAIT_GPIO echo "1" > /sys/class/gpio/gpio$AD5338R_RESET_PIN/value; WAIT_GPIO } term() { # cleanup all GPIOs if [ -d /sys/class/gpio/gpio$SX1302_RESET_PIN ] then echo "$SX1302_RESET_PIN" > /sys/class/gpio/unexport; WAIT_GPIO fi if [ -d /sys/class/gpio/gpio$SX1261_RESET_PIN ] then echo "$SX1261_RESET_PIN" > /sys/class/gpio/unexport; WAIT_GPIO fi if [ -d /sys/class/gpio/gpio$SX1302_POWER_EN_PIN ] then echo "$SX1302_POWER_EN_PIN" > /sys/class/gpio/unexport; WAIT_GPIO fi if [ -d /sys/class/gpio/gpio$AD5338R_RESET_PIN ] then echo "$AD5338R_RESET_PIN" > /sys/class/gpio/unexport; WAIT_GPIO fi } case "$1" in start) term # just in case init reset ;; stop) reset term ;; *) echo "Usage: $0 {start¦stop}" exit 1 ;; esac exit 0

Press  ctrl+x  to exit, press  y  to save changes, and then press  Enter  to return to the command line page.

- Modify the configuration code:

sudo vim ./tools/reset_lgw.sh

- Update the pin configurations:

SX1302_RESET_PIN=632 # SX1302 reset SX1302_POWER_EN_PIN=633 # SX1302 power enable SX1261_RESET_PIN=634 # SX1261 reset (LBT / Spectral Scan) # AD5338R_RESET_PIN=13 # AD5338R reset (full-duplex CN490 reference design)

Comment out lines 18, 29, 35, 42, 53, and 54 respectively:

...... # echo "$AD5338R_RESET_PIN" > /sys/class/gpio/export; WAIT_GPIO ...... # echo "out" > /sys/class/gpio/gpio$AD5338R_RESET_PIN/direction; WAIT_GPIO ...... # echo "CoreCell ADC reset through GPIO$AD5338R_RESET_PIN..." ...... # echo "0" > /sys/class/gpio/gpio$AD5338R_RESET_PIN/value; WAIT_GPIO # echo "1" > /sys/class/gpio/gpio$AD5338R_RESET_PIN/value; WAIT_GPIO

Press  ctrl+x  to exit, press  y  to save changes, and then press  Enter  to return to the command line page.

- replace the default SPI port of the LoraWAN®Module in the global_conf.json.sx1250.US915 config  file(Configuration files are selected based on the module you are using):

sudo nano packet_forwarder/global_conf.json.sx1250.US915

Modify the com_path parameter, change  "com_path": "/dev/spidev0.0" to  "com_path": "/dev/spidev2.0".

- Compile the code:

sudo make

These steps will configure LoRa® SPI and run the packet forwarder with the specified configuration file.

## 3.7.2 LoRa® USB

For LoRa® USB, the previous commands remain the same as for LoRa® SPI. However, the final command needs to be changed to:

cho 632 > /sys/class/gpio/export echo "out" > /sys/class/gpio/gpio632/direction echo "1" > /sys/class/gpio/gpio632/value sudo ./lora_pkt_fwd -c global_conf.json.sx1250.EU868.USB

This command specifies the configuration file to be used for LoRa® USB.

  - 3.8 5G Cellular over M.2 B-KEY

To interact with a 5G/4G module using AT commands via minicom, follow these steps:

- Create a new power_5g.sh file：

nano power_5g.sh

Open with sudo nano and enter the following command, then press  ctrl+x  to save and exit.

#!/bin/bash RESET_PIN=645 POWER_PIN=639 if [ ! -d "/sys/class/gpio/gpio$RESET_PIN" ]; then echo $RESET_PIN > /sys/class/gpio/export fi if [ ! -d "/sys/class/gpio/gpio$POWER_PIN" ]; then echo $POWER_PIN > /sys/class/gpio/export fi echo "out" > /sys/class/gpio/gpio$RESET_PIN/direction echo "out" > /sys/class/gpio/gpio$POWER_PIN/direction echo 1 > /sys/class/gpio/gpio$RESET_PIN/value echo 1 > /sys/class/gpio/gpio$POWER_PIN/value echo "Start to reboot 5g module" echo 0 > /sys/class/gpio/gpio$RESET_PIN/value sleep 0.05 echo 0 > /sys/class/gpio/gpio$POWER_PIN/value echo "5g module reboot completed"

- Execute the file:

sudo ./power_5g.sh

After 10-15 seconds (it takes a while for the module to power on and enumerate USB), check whether the device node appears:

ls /dev/ttyUSB*

Output /dev/ttyUSB0, etc.:

- Open minicom with the appropriate serial port and baud rate:

sudo apt update sudo apt install minicom sudo minicom -D /dev/ttyUSB2 -b 115200

This command opens minicom with the specified serial port (/dev/ttyUSB2) at a baud rate of 115200.

- Once minicom is open, you can start sending AT commands to the 4G module. For example:

AT

This command checks if the module is responsive. You should receive an "OK" response if the module is working properly.

- To dial a phone number using the 4G module, you can use the ATD command followed by the phone number:

ATD<phone_number>;

Replace <phone_number> with the desired phone number you want to dial.

Make sure to include a semicolon ; at the end of the command to indicate the end of the phone number.

  - 3.9 4G Cellular over M.2 B-KEY

Create a new power_4g.sh file:

sudo nano power_4g.sh

Open with sudo nano and enter the following command, then press ctrl+x to save and exit.

# SIM_MUX_SEL echo 643 > export echo out > gpio643/direction echo 0 > gpio643/value

Execute the file:

sudo ./power_4g.sh

After 10-15 seconds (it takes a while for the module to power on and enumerate USB), check whether the device node appears:

ls /dev/ttyUSB*

Output /dev/ttyUSB0.

Confirm the actual action of GPIO:

cat /sys/class/gpio/gpio645/value # should be 0 cat /sys/class/gpio/gpio639/value # should be 0

Both values are 0 → the script has been correctly pulled down and the module is in working state.

Enter minicom to send commands:

sudo minicom -D /dev/ttyUSB2 -b 115200

Press  Ctrl+A, Z , E  in sequence. First send AT to test whether it is connected. If OK appears, the connection is successful.

After executing the following command, the module will automatically restart. If you do not exit minicom, you can see the corresponding configuration information.

ECM Dial-up Internet Access:

AT+QCFG="usbnet",1

Until the last line shows OK, it will be successful.

Note

The device needs to wait for a while, and then you can view the ip address of usb0 in ifconfig.

Test network status and communication：

# Check network status ifconfig # Test communication ping www.baidu.com -I usb0

  - 3.10 Zigbee over Mini-PCIe

To test Zigbee communication with two Zigbee modules, follow these steps:

- Check Serial Ports:

Use the following command to check available serial ports:

cat /dev/ttyUSB*

- Install Serial Communication Tool:

sudo apt-get install cutecom

- Open Serial Port for Coordinator (First Zigbee Module):

Open the cutecom tool and configure it for the first serial port:

Baud rate: 115200

Check the "Hex output" option at the bottom of the interface.

Follow these steps to configure the first Zigbee module:

Set as coordinator: Send command ‘55 04 00 05 00 05’, expect response ‘55 04 00 05 00 05’.

Reset device: Press reset button or send command ‘55 07 00 04 00 FF FF 00 04’.

Network formation: Send command ’55 03 00 02 02‘.

- Open Serial Port for Router (Second Zigbee Module):

Open another instance of cutecom and configure it for the second serial port with the same settings as before.

Follow these steps to configure the second Zigbee module:

Set as router: Send command ‘55 04 00 05 01 04’, expect response ‘55 04 00 05 00 05’.

Reset device: Press reset button or send command ‘55 07 00 04 00 FF FF 00 04’.

Network formation: Send command ‘55 03 00 02 02’.

- Check Device Status:

Send command ‘55 03 00 00 00’ to check the device status. Expect a response similar to ‘55 2a 00 00 00 01 XX XX XX XX’, where ‘XX’ represents device information.

- Enter Transparent Mode:

If network formation is successful, enter transparent mode by sending command 55 07 00 11 00 03 00 01 13. Both modules should be in transparent mode for direct communication. To exit transparent mode, send "+++".

- Additional Notes:

- If router configuration fails, the device may already be a coordinator. Leave the network using command ‘55 07 00 04 02 xx xx xx’.

- Test transmission power using commands ’55 04 0D 00 00 0D‘ (query) and ’55 04 0D 01 XX XX‘ (set).

Ensure you replace /dev/ttyUSB* with the correct serial port for each Zigbee module. Follow these steps carefully to test Zigbee communication between the two modules successfully.

  - 3.11 RS485 Testing

reComputer Industrial R2000 includes 3x RS485 ports, and the corresponding COM ports and device files are as follows:

| Number of RS485 Ports | COM Port | Corresponding Silk Screen | Corresponding Device File |
| --- | --- | --- | --- |
| RS485-2 | COM2 | A2/B2/GND2 | /dev/ttyACM1 |
| RS485-3 | COM3 | A3/B3/GND3 | /dev/ttyACM2 |
| RS485-4 | COM4 | A4/B4/GND4 | /dev/ttyACM3 |

To test the RS485 function, you can follow the steps below (take RS485_1 and RS485_2 as examples):

- Please connect RS485_1 and RS485_2's A and B.

- Open minicom in two terminal windows respectively:

sudo minicom -D /dev/ttyACM1 sudo minicom -D /dev/ttyACM2

Note

If there is an expansion board, the number needs to be moved back one place, for example /dev/ttyAcM2 , /dev/ttyAcM3

- The following operations need to be performed on both opened ACMs:

  - Press ctrl+a, then press Z, and the Minicom Command Summary interface will appear:

  - Press O again to open configuration, select Serial port setup, and press Enter;

- Open all RS485 related interfaces, press H/I/J/K/L in sequence to open;

  - After all "YES" are displayed, press Enter to return, and then select Exit to exit.

Note

Take ACM2 and ACM3 as an example: If you want to send from ACM2 to ACM3, ACM2 needs to be set up again: ctrl+A, then press Z and then E , and then start the serial port write command. At this time, you can print strings in ACM2 at will, and you can see the contents of ACM2 in ACM3 at the same time; Conversely, if you want to send from ACM3 to ACM2, ACM3 needs to be set up again: ctrl+A, then press Z and then E , and then start the serial port write command. At this time, you can print strings in ACM3 at will, and you can see the contents of ACM3 in ACM2 at the same time. As shown in the figure:

  - 3.12 RS232 Testing

reComputer Industrial R2000 includes 1x RS232 ports, and the corresponding COM ports and device files are as follows:

| Number of RS485 Ports | COM Port | Corresponding Silk Screen | Corresponding Device File |
| --- | --- | --- | --- |
| RS232-1 | COM1 | RX1/TX1/GND1 | /dev/ttyACM0 |

Because RS232 is full-duplex communication, short-circuit the TX and RX of RS232 directly to perform a loopback test.

You need to open two terminals, ACM1 if the expansion board is connected, and ACM2 if the expansion board is not connected:

Terminal 1:

sudo minicom -D /dev/ttyACM1 -b 9600

If the expansion board is not connected, you need to change  /dev/ttyACM1  to /dev/ttyACM0 .

Terminal 2:

printf "hello seeed\r\n" > /dev/ttyACM1

Terminal 1 will display the content requested by Terminal 2 to be printed.

  - 3.13 DI Testing

reComputer Industrial R2000 contains 8x DI ports, user can configure these ports according to actual needs.

| Number of ports | DI ports | Corresponding extended GPIO |
| --- | --- | --- |
| 8 | DI1 | GPIO588 |
|  | DI2 | GPIO589 |
|  | DI3 | GPIO590 |
|  | DI4 | GPIO595 |
|  | DI5 | GPIO573 |
|  | DI6 | GPIO574 |
|  | DI7 | GPIO575 |
|  | DI8 | GPIO576 |

The input type of the DI ports is PNP. It supports input voltage is 5VDC~24VDC,current - 1000mA.

To test the functionality of DI, you can follow these steps to test it:

- The connection between the DI port of reComputer Industrial R2000 and the external load has been completed.

- Enter the following command to get the status of GPIO：

echo 588 > /sys/class/gpio/export echo in > /sys/class/gpio/gpio588/direction cat /sys/class/gpio/gpio588/value

- 3、 When the external level is high, the value of  /sys/class/gpio/gpio588/value  is 0; when the external level is low,  /sys/class/gpio/gpio588/value  is 1.

  - 3.14 DO Testing

reComputer Industrial R2000 contains 8x DO ports, user can configure these ports according to actual needs.

| Number of ports | DI ports | Corresponding extended GPIO |
| --- | --- | --- |
| 8 | DO1 | GPIO638 |
|  | DO2 | GPIO637 |
|  | DO3 | GPIO636 |
|  | DO4 | GPIO635 |
|  | DO5 | GPIO577 |
|  | DO6 | GPIO578 |
|  | DO7 | GPIO594 |
|  | DO8 | GPIO596 |

The output type of the DO ports is transistor. It supports output voltage - under 60 VDC, current capacity - 500 mA.

To test the functionality of DO, you can follow these steps to test it:

- The connection between the DO port of reComputer Industrial R2000 and the external load has been completed.

- Enter the following command to set the output to high level or low level：

echo 638 > /sys/class/gpio/export echo out > /sys/class/gpio/gpio638/direction echo 1 > /sys/class/gpio/gpio638/value echo 0 > /sys/class/gpio/gpio638/value

- When the external level is high, the value of  /sys/class/gpio/gpio638/value   is 0; when the external level is low,  /sys/class/gpio/gpio638/value  is 1.

  - 3.15 USB Hub Testing

To test the USB hub, you can use the following steps:

- Check if the USB hub is detected by running the lsusb command. This command lists all connected USB devices, including hubs.

lsusb

Running this command should display information about the USB devices connected to your system, including any USB hubs that are present.

If the USB hub is functioning properly, you should see its details listed in the output of the lsusb command. If it's not listed, there may be an issue with the hub or its connection to the system. In such cases, you may need to troubleshoot the USB hub or its connections.

  - 3.16 RTC

To test the Real-Time Clock (RTC) functionality, follow these steps:

- Disable automatic time synchronization:

sudo systemctl stop systemd-timesyncd sudo systemctl disable systemd-timesyncd

- Set the time :

sudo hwclock --set --date "2025-7-17 12:00:00"

- Synchronize the RTC time to the system:

sudo hwclock --hctosys

- Check the RTC time:

sudo hwclock -r

This command will read and display the time stored in the RTC.

- Disconnect the power source from the RTC, wait a few minutes, then reconnect it and check the RTC time again to see if it retained the correct time.

  - 3.17 Watchdog

To perform a watchdog test, follow these steps:

- Install the watchdog software:

sudo apt install watchdog

- Edit the watchdog configuration file:

# make sure you install vim already, if haven't, can install by the command below sudo apt-get install vim sudo vim /etc/watchdog.conf

Modify the configuration as follows:

watchdog-device = /dev/watchdog # Uncomment and edit this line for hardware timeout values that differ # from the default of one minute. watchdog-timeout = 120 # If your watchdog trips by itself when the first timeout interval # elapses then try uncommenting the line below and changing the # value to 'yes'. #watchdog-refresh-use-settimeout = auto # If you have a buggy watchdog device (e.g. some IPMI implementations) # try uncommenting this line and setting it to 'yes'. #watchdog-refresh-ignore-errors = no # ====================== Other system settings ======================== # # Interval between tests. Should be a couple of seconds shorter than # the hardware time-out value. interval = 15 max-load-1 = 24 #max-load-5 = 18 #max-load-15 = 12 realtime = yes priority = 1

You can adjust other settings as needed.

- Ensure the watchdog service is running:

sudo systemctl start watchdog

- To test the watchdog functionality, execute the following command to simulate a system hang:

sudo su echo 1 > /proc/sys/kernel/sysrq echo "c" > /proc/sysrq-trigger

This command triggers a kernel crash and should cause the watchdog to reboot the system.

- Monitor the system to confirm that it reboots after the specified timeout period.

These steps will help you test and ensure the functionality of the watchdog timer on your system.

  - 3.18 Buzzer

The GPIO corresponding to the buzzer is gpio627. Enter the following script to turn the buzzer on/off :

- Turn on the buzzer :

echo 627 > /sys/class/gpio/export echo out > /sys/class/gpio/gpio627/direction echo 1 > /sys/class/gpio/gpio627/value

- Turn off the buzzer :

echo 627 > /sys/class/gpio/export echo out > /sys/class/gpio/gpio627/direction echo 0 > /sys/class/gpio/gpio627/value

  - 3.19 TPM 2.0

If you connect TPM 2.0 module to device, the following code can help check TPM connection.

ls /dev ¦ grep tpm

If you see tpm0 and tpmrm0 in the output, it means that TPM (Trusted Platform Module) devices are detected and available on your system. This indicates that the TPM hardware is recognized and accessible, which is a good sign. You can proceed with using TPM-related functionalities or applications knowing that the devices are present and accessible.

  - 3.20 ATECC608A

To interact with the ATECC608A device and generate a random serial number, follow these steps:

- Clone the atecc-util Repository:

curl -LJO https://github.com/wirenboard/atecc-util/releases/download/v0.4.12/atecc-util_0.4.12_arm64.deb

- Extract the contents of the .deb package to the current directory:

dpkg -x ./atecc-util_0.4.12_arm64.deb .

- Navigate to the atecc Directory:

cd usr/bin

- Generate a Random Serial Number:

./atecc -b 10 -s 192 -c 'serial'

-b 1  → Uses slot 1.

-s 192  → Sets the serial number size to 192 bits.

-c 'serial'   → Generates a random serial number.

This command instructs the ATECC utility to use slot 1 (-b 1), set the serial number size to 192 bits (-s 192), and generate a

random serial number (c 'serial'). The output will be the generated serial number, such as "01235595d3d621f0ee".

This process allows you to interact with the ATECC608A device and perform various operations, such as generating random serial numbers.

  - 3.21 EEPROM

Here are the commands to interact with an EEPROM (Electrically Erasable Programmable Read-Only Memory):

- Grant full permissions (read, write, and execute) to the EEPROM device file:

sudo chmod 777 /sys/bus/i2c/devices/10-0050/eeprom

- Write the string "This is a test string" to the EEPROM device:

echo "This is a test string" > /sys/bus/i2c/devices/10-0050/eeprom

- Read the contents of the EEPROM device and displays it in hexadecimal format using the hexdump utility:

sudo cat /sys/bus/i2c/devices/10-0050/eeprom ¦ hexdump -C

  - 3.22 SSD

To list the disks, including the SSD, you can use the fdisk -l command. Here's how:

sudo fdisk -l

This command will display a list of all disks connected to your system, including the SSD if it's properly detected. Look for entries that represent your SSD. They typically start with /dev/sd followed by a letter (e.g., /dev/sda, /dev/sdb, etc.).

Once you identify the entry corresponding to your SSD, you can proceed with partitioning or formatting it as needed.

  - 3.23 UPS for Safe Shut Down

A GPIO6 between CPU and DC power in is used to alarm CPU when the power supply is down. Then the CPU should do something urgent in a script before energy exhaustion of super capacitor and run a "$ shutdown".

Another way to use this function is Initiate a shutdown when GPIO pin changes. The given GPIO pin is configured as an input key that generates KEY_POWER events. This event is handled by systemd-logind by initiating a shutdown.

- Hardware connection.

Please make sure that the `CM5_UPS_DET` pin of the UPS device is connected to the GPIO16 pin of the R2000 device.

- Modify the configuration file.

  - Open the terminal.

  - Execute the following command to edit the configuration file:

sudo nano /boot/firmware/config.txt

- Add the following content at the end of the file:

dtoverlay=gpio-shutdown,gpio_pin=GPIO16,active_low=1

Save and exit the editor (press `Ctrl+O` to save, `Enter` to confirm, and `Ctrl+X` to exit).

- Prepare Python script

  - Create a new Python script file:

cd ~ sudo nano ups_shutdown.py

  - Copy and paste the following code into the file:

import RPi.GPIO as GPIO import time, os num = 0 GPIO.setmode(GPIO.BCM) # Set GPIO16 to input mode # Add 500ms anti-shake time to stabilize the software GPIO.setup(16, GPIO.IN, pull_up_down=GPIO.PUD_UP) GPIO.add_event_detect(16, GPIO.FALLING, bouncetime=500) while True: if GPIO.event_detected(16): print("...External power off...") print("") # Sync data to disk os.system('sync') print("...Data saving...") print("") # Sleep for 3 seconds time.sleep(3) # Synchronize data again os.system('sync') # Countdown 5 seconds while num < 5: print('----------') s = 5 - num print('---' + str(s) + '---') num = num + 1 time.sleep(1) print('----------') # Execute shutdown command os.system('sudo shutdown -h now')

- Save and exit the editor (press `Ctrl+O` to save, `Enter` to confirm, and `Ctrl+X` to exit).

- Run the script.

  - Open the terminal.

  - Execute the following command to run the script:

sudo python3 ups_shutdown.py

Note

Use `sudo` to ensure that the script has sufficient permissions to execute the shutdown command.

- Simulate power failure test

  - Cut off the external power supply.

  - Observe whether the system automatically saves data and shuts down.

- Verify the result

  - Reconnect the power supply.

  - Check whether the system data is complete and starts normally.

Note

1. For UPS function please contact us for more information. 2. The alarm signal is active LOW.

  - 3.24 AI  Accelerator

The M.2 M-KEY 2280 slot on the reComputer Industrial R2000 is designed to accommodate PCIE M.2 AI  Accelerator. And the R2000-12 series has been pre-installed with a Hailo-8 M.2 AI Acceleration up to 26TOPS.

The device comes pre-installed with the Hailo accelerator driver, so you can use it directly and run the test case:

- Navigate to the test case directory

cd /mnt/hailo-rpi5-examples/

- Start the virtual environment

source ./setup_env.sh

- Run the simple detection example

python basic_pipelines/detection_simple.py

To close the application, press `Ctrl+C` .

This is lightweight version of the detection example, mainly focusing on demonstrating Hailo performance while minimizing CPU load. The internal GStreamer video processing pipeline is simplified by minimizing video processing tasks, and the YOLOv6 Nano model is used.

Note

If the reComputer you purchased does not include Hailo-8 and you are considering purchasing a Hailo device for integration, please refer to the official Hailo documentation (https://github.com/hailo-ai) to configure the firmware and environment, and run the examples to verify that the device can be used normally.

  - 3.25 Installing Ubuntu on reComputer Industrial R2000

- Burn Image

- Follow the instructions provided on the https://ubuntu.com/download/raspberry-pi to burn the downloaded image.

- Initial Setup:

- Connect a monitor to your Raspberry Pi.

- Power on the Raspberry Pi.

- Initial Configuration:

- Follow the onscreen instructions for the initial setup. This may include setting up the user account, 
language, and other preferences.

- Install Seeed provided firmware:

- Once logged into the system, open a terminal window.

- Clone the Seeed-Studio GitHub repository by running the following command:

git clone --depth 1 https://github.com/Seeed-Studio/seeed-linux-dtoverlays.git

Navigate to the cloned repository directory:

cd seeed-linux-dtoverlays

Run the provided script to configure the display device. For example, to configure for reComputer-R2x, run:

sudo ./scripts/reTerminal.sh --device reComputer-R2x

- Reboot:

After running the script, reboot your reComputer Industrial R2000 by running:

sudo reboot

  - 3.26 Customized Linux: Yocto and Mender

The reComputer Industrial R2000 is an edge IoT controller device that utilizes the Raspberry Pi CM5 as its processor. When you require a customized Linux distribution to run on your device, as well as a convenient solution for managing software updates, Yocto Project and Mender come into play.

Yocto Project is a powerful tool tailored for creating custom Linux distributions specifically designed for embedded devices, ensuring that your device's requirements are met. On the other hand, Mender serves as an open-source over-the-air (OTA) software update manager for embedded Linux devices, simplifying the process of managing software updates. It enables remote management of software updates via the internet, eliminating the need for physical access to the device and ensuring its security and stability.

Therefore, we provide the official reComputer R110X script along with Yocto Project-generated images, as well as scripts for Mender updates, for your reference and convenience.

Note

More details and tutorial please refer to https://wiki.seeedstudio.com/recomputer_Industrial R2000_yocto_mender/

  - 3.27 Costomized Linux: Buildroot

Buildroot is a powerful tool for developers who want to create a customized and lightweight Linux environment tailored to their embedded device's hardware and software requirements.Buildroot is another tool similar to Yocto Project that can be used to create custom Linux distributions for reComputer Industrial R2000. It is less resource-intensive than Yocto Project, making it a suitable choice for devices with limited processing power or memory. It is recommended to consult the Buildroot documentation and community resources to determine its compatibility with the device.

Note

More details and tutorial please refer to https://wiki.seeedstudio.com/recomputer_Industrial R2000_buildroot/

- To download and compile the Seeed Studio Linux Buildroot code, you may need to install the following libraries if they are not already installed:

sudo -E apt-get install sed make binutils build-essential gcc g++ bash patch gzip bzip2 perl tar cpio unzip rsync file bc wget python cvs git mercurial subversion

- clone the code from github

git clone --depth 1 https://github.com/Seeed-Studio/seeed-linux-buildroot.git

- For 32-bit or 64-bit systems, configure and compile using predefined configuration files:

cd seeed-linux-buildroot # For 64-bit: make reComputer_R2x_64_defconfig # For 32-bit: make reComputer_R2x_32_defconfig make

The final firmware files will be located in the following path: seeed-linux-buildroot/output/images/.

C4. Assembly Guide

  - 4.1 Disassembly Guide

Following these steps should help you disassemble the device without any issues.

Step 1: Remove the Four Screws at the Bottom and remove the front and rear panels:

- Locate and unscrew the four screws located at the bottom of the device using an appropriate screwdriver.

- Once the screws are removed, carefully lift off the front and rear panels from the device

Step 2:  Unscrew the side antenna connector nuts and remove the Floor Panel.

Step 3: Remove the four screws that secure the PCB in place

  - 4.2 Assemble Nano SIM Card

Step 1: Load the Nano SIM Card into the SIM slot.

  - 4.3 Assemble SSD

Step 1: Remove the back cover following the disassembly guide.

Step 2: Load the SSD into the M.2 socket and lock the screws.

  - 4.4 Assemble M.2 AI  Accelerator

Step 1: Disassemble the entire device following section 4.1 "Disassembly Guide".

Step 2: Load the AI Accelerator into the M.2 socket and lock the screws.

- 4.5 Assemble 5G/4G/LoRa®/Zigbee Module and Antenna

Step 1: Remove the Expantion Board and load the 4G module/LoRa® Module/Zigbee Module into the Mini-PCIe/M.2 B-KEY slot and lock the screws.

Step 2:Connect the feeder to the antenna hole on the housing as shown in the following diagram.

Step 3: Install the feeder into the antenna base of the corresponding module

  - 4.6 Assemble TPM 2.0 Module

Step 1: Remove the back cover following the disassembly guide.

Step 2: Load the TPM 2.0 module into the J26 socket.

  - 4.7 Assemble UPS module

Step 1: Before installing the UPS and PoE module on the CM5 module side of board, disassemble the entire device following the disassembly guide provided.

Step 2:

- Using two PM2.0xL5.0 screws and M2.0x5.0 standoffs, secure the UPS module onto two holes without metal contact pads.

- Make sure the UPS module is aligned properly and firmly attached using the provided screws and standoffs.

Step 3: Install the PoE Module

- Align the PoE module with the designated aperture on the board.

- Carefully solder the PoE module onto the board. Due to the compact nature of the board, exercise caution while soldering to avoid damaging nearby components.

- 4.8 Mounting Guide

4.8.1 DIN-rail Mounting Guide

reComputer Industrial R2000 offers various installation methods. The DIN-rail clip and installation screws are included in the packaging. Follow the diagram to correctly attach the DIN-rail clip to the mounting holes on the side of the device. Once the screws are securely fastened, you can then install the device onto the mounting rail.

Step 1: Place the device and rail clip on the upper edge of the standard profile rail at the position  shown and push the device down.

Step 2: Swing the rail clip of the device from below through the standard profile rail.

Step 3: Push the device in the direction of the standard profile rail. You will hear the device click into  place.

Removing

Step 1: Push down the device until it is released by the rail clip.

Step 2: Swing the device out of the standard profile rail.

Step 3: Lift the device up and of

4.8.2 Wall Mounting Guide

Vertical mounting method is also suitable for reComputer Industrial R2000, however the mounting brackets are not included in box, that need additional purchase.

Step 1: Lay the mounting brackets on the rear of the device.

Step 2: Fasten the brackets with supplied screws.

Step 3: Mark the bore holes, drill the required holes in the wall and fasten the device to the wall using two screws.

C5. Accessories List

| Item | Product | Product Name | SKU |
| --- | --- | --- | --- |
| LoRa® module |  | Region optional LoRaWAN Gateway Module(SPI)-US915 | 114992969 |
|  |  | Region optional LoRaWAN Gateway Module(SPI)-EU868 | 114993268 |
|  |  | Region optional LoRaWAN Gateway Module(USB)-US915 | 114992991 |
|  |  | Region optional LoRaWAN Gateway Module(USB)-EU868 | 114992628 |
| LoRa® Antenna |  | LoRa Antenna Kit - 868-915 MHz | 110061501 |
| Wi-Fi/BLE Antenna |  | Raspberry Pi Compute Module 4 Antenna Kit | 114992364 |
| 4G Module |  | LTE Cat 4 EC25-AFXGA-Mini-PCIe Module - for North American | 113991134 |
|  |  | LTE Cat 4 EC25-EUXGR-Mini-PCIe Module - for EMEA and Thai | 113991135 |
|  |  | LTE Cat 4 EC25-AUXGR-Mini-PCIe Module - for Australia | 113991174 |
|  |  | LTE Cat 4 EC25-EFA-Mini-PCIe Module - for Thai | 113991214 |
|  |  | LTE Cat 4 EC25-EMGA-Mini-PCIe Module - for Malaysia | 113991234 |
|  |  | LTE Cat 4 EC25-JFA-mini-PCIe | 113991296 |
| 4G Antenna |  | 4G Antenna Kit for 4G module | 110061502 |
| GPS Antenna |  | GPS Antenna Kit for EC25 4G Module | 110061521 |
| UPS Supercapacitor |  | SuperCAP UPS LTC3350 Module | 110992004 |
| Encryption chip TPM 2.0 |  | TPM 2.0 Module with infineon SLB9670 | 114993114 |
| SSD Card |  | NVMe M.2 2280 SSD 2TB | 114993467 |
|  |  | NVMe M.2 2280 SSD 1TB | 112990267 |
|  |  | 512GB NVMe M.2 PCle Gen3x4 2280 Internal SSD | 112990247 |
|  |  | 256GB NVMe M.2 PCle Gen3x4 2280 Internal SSD | 112990246 |
|  |  | 128GB NVMe M.2 PCle Gen3x4 2280 Internal SSD | 112990226 |

C6. Warranty & Support

  - 6.1 Warranty

1. From the date of sale, the company provides 24 months free warranty for the products.

2. Warranty coverage is limited to products purchased from the official Seeed Studio website or authorized distributors. Customers need to keep receipts and purchase vouchers.

3. The products to be repaired shall be properly packaged and transported, and the customer shall be responsible for any loss or damage during transportation.

4. During the warranty period, the freight and maintenance costs arising from product quality failures shall be borne by Seeed Studio. If the warranty period exceeds 24 months, Seeed Studio will charge the fee for replacing parts according to the product failure, and the freight is borne by the user.

5. During the free warranty period, in case of any of the following events, Seeed Studio has the right to refuse service or charge materials and service fees at its discretion.

Product failure or damage caused by improper use by users.

The product label is damaged and the product information cannot be identified.

Even within the warranty period, if the product has functional issues or is difficult to repair due to improper customer use, unaut- horized disassembly or modification, poor operating environment, improper maintenance, accidents, or other reasons. Seeed Studio reserves the right to make judgments on the above situations and collect maintenance fees.

Other unavoidable external factors cause product failure and damage.

The above warranty regulations are only applicable to the above Seeed Studio reComputer Industrial R2000 series, other products are not applicable!

- 6.2 Support

Quick start guide:

https://wiki.seeedstudio.com/recomputer_r/

Tech support email:

If you encounter any issues while deploying or testing, please don't hesitate to contact our technical support team at techsupport@seeed.io, or refer to our online knowledge base, https://wiki.seeedstudio.com.

Customized service email:

For further information about customizations, welcome you to directly reach out at edge@seeed.cc, we will provide prompt reply.

Discord:

Discord community:

Welcome to join our official community, where you can exchange product-related questions and get relevant support.

https://discord.seeed.cc